ESS Section 5

Contents:

5. Analog PID Controllers

5.1 Locus of Poles and Stability

5.1.1  Linear Feedback-Loop Setup

5.1.2  Locus of System Poles and Closed-Loop Stability

5.1.3  Open-Loop Gain’s Phase Margin an Closed-Loop Stability

5.1.4  Impact of Delay

5.2 Passive Loop Gain Compensation

5.2.1  Stabilization by Lowering the Amplification

5.2.2  Stabilization by Adding a Low-Frequency Pole

5.2.3  Stabilization by Adding a Zero Onto the Cross-Over Frequency

5.2.4  Using Pole Splitting for Phase Margin Compensation

5.2.5  Lead-Lag Compensation

5.3 Active PID Loop Gain Compensation

5.3.1  Stabilization by an Active Differentiator

5.3.2  Compensation Using a Differentiator with Pole

5.3.2  Stabilization by Constant Gain

5.3.3  Advantages of an Integrator

5.4 Time-Continuous PID Compensation

5.4.1  Simplest PID Compensator Model

5.4.2  Compensation Using a Differentiator with Pole

5.4.3  PID Compensator Model with Pole Tdp

5.4.4  PID Compensator Model with Poles ωip and Tdp

5.4.5  PID Compensator Application to a DC/DC Buck Converter

Procedure. In class, this MS Word document is written using a tablet PC. Page breaks and/or subsection headings can be moved. After class, students are offered both the modified Word document and a PDF printout of it.

# Analog PID Controllers

### Linear Feedback-Loop Setup

| - Closed loop with error entry E. |  |
| --- | --- |
| - Opened loop and test signal feed-in A. Here, phase margin ΦM must be measured against -360° = 0°, as the inversion is part of the open loop now. |  |
| Fig. 5.1.1: Linear feedback loop systems |  |

Table 5.1.1: Symbols, their acronyms and meanings, definitions, formulae

| Signal | Significance |  |  |
| --- | --- | --- | --- |
| A | test signal intput |  |  |
| C | Compensator output signal |  |  |
| E | Error entry |  |  |
| ε | Setpoint – actual value aberration |  |  |
| P | Plant/Process output signal |  |  |
| V | process variable |  |  |
| X | input signal |  |  |
| Y | output signal |  |  |
| Symbol | Significance / Network Functionality | Definition | Formula |
| B | feedBack network transfer function |  |  |
| CTF | Compensator Transfer Function |  | (typically PID) |
| F | Forward network transfer function |  |  |
| FB, LG | (open) Loop Gain |  |  |
| NTF | Noise Transfer Function |  |  |
| PTF | Plant/Process Transfer Function |  |  |
| QTF | Inference (Quarrel) input Transfer Function |  |  |
| STF | Signal Transfer Function |  |  |

### Locus of System Poles and Closed-Loop Stability

Fig. 5.1.2 (a) Pole-paires of Closed-loop gain,                (b) corresponding step responses.

The homogeneous system (i.e. without excitation) behaves according to

,

The second order system as a function cut-off frequency ω0, DC-amplification A0 and damping constant D, is written with S=s/ω0 in a common form as

H(s) = =i=1Naiespit= = =i=1Naiespit,

### Open-Loop Gain’s Phase Margin and Closed Loop Stability

| Fig. 5.1.2.1: Frequency compensaton by setting phase margin to M45°: - Amplitude diagram with (dashed) and without (solid line) compensation. - Phase diagram with (dashed) and without (solid line) compensation. |  |
| --- | --- |

Simulation example with 45° phase margin

Fig. 5.1.2.2 : Compensation goal: the second pole, here named ωp1, is placed onto cross-over frequency ωx1 of open-loop gain FB1, to yield phase margin of ΦM = 45°. Figure parts:	
upper left: impulse response of STF1, lower left: step response of STF1,	
right: Bode diagram of blu: F1=CTF1∙PTF1, green: CTF1, pink: B1, yellow: STF1.

Fig. 5.1.2.2 generated by the code in listing 5.1.2.2 illustrates a Matlab simulation of 2nd-order system with the 2nd pole, here named ωp1, is placed onto cross-over frequency ωx1 of the open-loop gain FB1, to yield phase margin of ΦM = 45°.

Listing 5.1.2.2: generates Matlab plot Fig. 5.1.2.2

%% 1st order system

b1   = 0.1;    % feedback network as constant factor

B1   = tf(b1); % feedback network as transfer function

wx1  = 0.1; % cross-over frequency of FB1 without compensation

wcp  = wx1;               % pole to be set

CTF1 = tf([1],[1/wcp 1]); % set a pole at wcp

A1   =1e5; wp1=1e-5;         % DC amplification and 1. pole of plant

PTF1 = tf([A1*wp1],[1 wp1]); % plant / process transfer function

F1   = CTF1*PTF1; % total forward network

FB1  = F1*B1; % open loop gain

STF1 = feedback(F1,B1); % signal transfer function of the closed loop

figure(1); % 1st order sysmtem's graphical postprocessing > figure 1

subplot(221); impulse(STF1); grid on;

subplot(223); step(STF1); grid on;

subplot(122); bodeplot(F1,FB1,STF1,B1,CTF1); grid on;

xlim([1e-7 1e2]);

### The Impact of Delay

A delay Tdel in time domain translates to phase shift  Φdel = - ω∙Tdel  in frequency domain:

with FT symbolizing the fourier transform. Proof using τ = t – T    dτ/dt = 1    dt = dτ :

This is a linar phase response:

At cross-over frequency ωx , marked by unitiy loop-gain |FB(ωx)| = 1, this delay causes a phase margin loss of

.

On a logharithmic frequency axis the phase loss Φdel seems to behave exponential.

## Passive Loop Gain Compensation Techniques

### Stabilization by Lowering the Amplification

### Stabilization by Adding a Low-Frequency Pole

### Stabilization by Adding a Zero Onto the Cross-Over Frequency

Fig. 5.2.2 : setting a zero ωn2 at ωx2:	
upper left: impulse response of STF2, lower left: step response of STF2,	
right: Bode diagram of blu: F2=CTF2∙PTF2, green: CTF2, pink: B2, yellow: STF2.

Fig. 5.2.2 generated by the code in listing 5.2.1 illustrates a Matlab simulation of introducing a pole on the cross-over frequency ωx2.

Listing 5.2.2: generates Matlab plot Fig. 45.4.2.2

%% 2nd order system

b2   = 0.1;    % feedback network as constant factor

B2   = tf(b2); % feedback network as transfer function

wx2  = sqrt(b2); % cross-over frequency of FB2 without compensation

wcn  = wx2;               % zero to be set

CTF2 = tf([1/wcn 1],[1]); % set a zero at wcn

A2   = 1e6; wp2=1e-3; D2=1; % DC ampl., double-pole, damp. const. of plant

PTF2 = tf([A2*wp2*wp2],[1 2*D2*wp2 wp2^2]);

F2   = CTF2*PTF2; % total forward network

FB2  = F2*B2; % open loop gain

STF2 = feedback(F2,B2); % signal transfer function of the closed loop

figure(2); % 2nd order sysmtem's graphical postprocessing > figure 2

subplot(221); impulse(STF2); grid on; %upper left quarter: impulse response

subplot(223); step(STF2); grid on;    %lower left quarter: step response

subplot(122); bode(F2,FB2,STF2,B2,CTF2); grid on; % right half: Bode diag.

xlim([1e-6 1e2]); % set abscissa range

### Using Pole Splitting for Phase Margin Compensation

### Lead-Lag Compensation

Fig. 5.2.4.1 (a) introducing a zero at fn = fx / a, 	(b)  zero at fdn = fx / a and pole at fdp = fx∙a3.

Figure part (a) uses a single zero to stabilize a 2nd order system with uncompensated and compensated cross-over frequencies fx and fy, respectively. For a > 1 we get fn∙a = fx =fy / a.

Figure part (b) uses a lead-lag compensator to stabilize a 2nd order system with uncompensated and compensated cross-over frequencies fx and fy, respectively. For a > 1 delivers fy = a∙fx with zero-pole pair fdn∙a2 = fy =fdp / a2, corresponding to fdn∙a = fx =fdp / a3.

Fig. 5.2.4.2 : introducing a zero at ωcn = ωx3 / a3 and pole at ωcp = ωx3∙(a3)3 with a=sqrt(10):	
upper left: impulse response of STF3, lower left: step response of STF3,	
right: Bode diagram of blu: F3=CTF3∙PTF3, green: CTF3, pink: B3, yellow: STF3.

Fig. 5.2.4.2 generated by the code in listing 5.2.4.2 illustrates a Matlab simulation of introducing a lead-lag element around the cross-over frequency ωx3.

Listing 5.2.4.2: generates Matlab plot Fig. 5.2.4.2

%% Lead-Lag Element in 2nd order system

b3   = 0.01;   % feedback network as constant factor

B3   = tf(b3); % feedback network as transfer function

wx3  = sqrt(b3); % cross-over frequency of FB2 without compensation

a3   = sqrt(10); % user's choice to set a3 | sqrt(10) > 7% volt. overshoot

wcn  = wx3/a3; % zero to be set

wcp  = wx3*a3^3; % zero to be set

CTF3 = tf([1/wcn 1],[1/wcp 1]); % lead-lag: set zero at wcn and pole at wcp

A3   = 1e8; wp3=1e-4; D3=1; % DC ampl., double-pole, damp. const. of plant

PTF3 = tf([A3*wp3*wp3],[1 2*D3*wp3 wp3^2]);

F3   = CTF3*PTF3; % total forward network

FB3  = F3*B3; % open loop gain

STF3 = feedback(F3,B3); % signal transfer function of the closed loop

figure(3); % 2nd order sysmtem's graphical postprocessing > figure 2

subplot(221); impulse(STF3); grid on; %upper left quarter: impulse response

subplot(223); step(STF3); grid on;    %lower left quarter: step response

subplot(122); bode(F3,FB3,STF3,B3,CTF3); grid on; % right half: Bode diag.

xlim([1e-6 1e2]); % set abscissa range

Christmas and new Year Break 2022

## Active PID Loop Gain Compensation

### Stabilization by an Active Differentiator

### Compensation Using a Differentiator with Pole

### Stabilization by Constant Gain

### Advantages of an Integrator

## Time-Continuous PID Compensation

### Simplest PID Compensator Model

The most simple PID compensator model delivers

and on a single fraction bar

Disadvantages are stability problems and the fact, that numerator order > denominator order is not possible, the system was non-causal and would deliver infinite amplification for infinite frequencies.

### PID Compensator Model with Pole Tdp

Typically, we find a mathematical model of the form

,

which delivers on a single fraction bar

.

Relationships to Matlab / Simulink control parameters P, I, D, N are:

### PID Compensator Model with Poles ωni and Tdp

Fig. 5.4.3: PID Compensator, (a) topology and (b) asymptotic Bode diagram

Although mathematical models typically use ωip = 0, a finite ωip > 0 is inevitable for analog amplifiers, and in the digital domain the range of representable numbers is limited. The three-term model

delivers on a single fraction bar

which represents the 2nd order LTI system

with coefficients

Relationships between PID parameters and corner frequencies in Fig. part (b) are

Relationships to Matlab / Simulink control parameters P, I, D, N are:

### PID Compensator Application to a DC/DC Buck Converter

(a) red: TF of RLC lowpass, black target loop gain,  (b) same as (a), illustrate compensator

Fig. 5.4.4: PID Compensator, (a) target loop-gain,  (b) illustrate compensator asymptotes

References

- Wikipedia, Phase margin, available: https://en.wikipedia.org/wiki/Phase_margin